3EJ4 Formula Sheet

# M0: Review of 2EI4

## MOSFET DC/Large-Signal:

VOV=VGS-Vtn=VSG-Vtp 		K=K'WL=μeCoxWL  [A/V2]

| Region of Operation (NMOS) | Assume | Check |
| --- | --- | --- |
| Cutoff | IDS=0 | VGS≤Vtn |
| Linear | IDS=KnVOVVDS-VDS22 | VGS≥Vtn 0≤VDS≤VOV |
| Saturation | IDS=Kn2VOV2(1+λVDS) | VGS≥Vtn VDS≥VOV≥0 |

**For DC analysis, unless explicitly specified otherwise, assume CLM effect λ=0

**For PMOS, Vtp<0. To prevent the sign conversion use Vtp.

| Region of Operation (PMOS) | Assume | Check |
| --- | --- | --- |
| Cutoff | ISD=0 | VSG≤Vtp |
| Linear | ISD=KpVOVVSD-VSD22 | VSG≥Vtp 0≤VSD≤VOV |
| Saturation | ISD=Kp2VOV2(1+λVSD) | VSG≥Vtp VSD≥VOV≥0 |

## BJT DC/Large-Signal:

VA=1λ		IE=IB+IC

| Region of Operation (NPN) | Assume | Check |
| --- | --- | --- |
| Cutoff | IB=IC=0 | VBEon<0.7V VBC(on)<0.5V |
| Saturation | VBEon=0.7V VBC(on)=0.5V | IB,IC>0 IC≤βIB |
| Active | VBEon=0.7V IC=βIB1+VCEVA | IB>0 VCE≥0.3V VBC(on)<0.5V |

**For DC analysis, unless explicitly specified otherwise, assume VA=∞

| Region of Operation (PNP) | Assume | Check |
| --- | --- | --- |
| Cutoff | IB=IC=0 | VEBon<0.7V VCB(on)<0.5V |
| Saturation | VEBon=0.7V VCB(on)=0.5V | IB,IC>0 IC≤βIB |
| Active | VEBon=0.7V IC=βIB1+VECVA | IB>0 VCE≥0.3V VCB(on)<0.5V |

## MOSFET Small Signal Model Parameters:

gm=2IDSVOV=2KID(1+λVDS)≈2KID [S]  		ro=1λ+VDSIDS ≈1λIDS ≈VAIDS 	re=1gm

## BJT Small Signal Model Parameters:

gm=ICVT , VT=25mV	rπ=βgm   	re=αgm=VTIE       	ro=VAIC		α=ββ+1

ic=β∙ib=α∙ie	ie=ib+ic=(β+1)ib

## Small Signal Models:

π-model

T-model

**The π-model is used when the s/e is AC grounded.

**The T-model is used when g/b is AC grounded.

## MOSFET Single Stage Transistors

## BJT Single Stage Transistors

# M1: Linear Two-Port Network Theory

## Three Amplifier Parameters

Avo=voviRL=∞ (open circuit voltage gain)	Ro=vxix (output resistance)

Rin=viii (input resistance)

## Three Parameter Amplifier Model

**Used when analyzing the overall amplifier block i.e. when RL/Rsig present.

## Voltage Gains of Amplifiers

Av=vovi (amplifier voltage gain including RL)

Gv=vovsig (overall voltage gain including RL and Rsig)

## Two-Port Network Parameters:

**Used when Rin not function of RL and Rsig.

| Impedance (z) Parameters: v1=z11i1+z12i2 v2=z21i1+z22i2 | Admittance (y) Parameters: i1=y11v1+y12v2 i2=y21v1+y22v2 |
| --- | --- |
| Hybrid (h) Parameters: v1=h11i1+h12v2 i2=h21i1+h22v2 | Inverse-Hybrid (g) Parameters: i1=g11v1+g12i2 v2=g21v1+g22i2 |

| z | y | h | g |
| --- | --- | --- | --- |
| Ro=z22 | Ro=1/y22 | Ro=1/h22 | Ro=g22 |
| Ri=z11 | Ri=1/y11 | Ri=h11 | Ri=1/g11 |
| Azo=z21 | Ago=y21 | Aio=h21 | Avo=g21 |
| Rin=viii | Yin=iivi | Rin=viii | Yin=iivi |

# M2: Differential Amplifiers

## Definition

νDM=νG1-νG2	νCM=νG1+νG22

νG1=νCM+νDM2	νG2=νCM-νDM2

## Differential Pair Large Signal Analysis

For MOS: VDM,max=2VOV	vdm,max=VOV2

For BJT: VDM,max=4VT≈100 mV	vdm,max=VT2≈12.5 mV

## DC Input Common Mode Range

Highest VCM is limited by Q1,2 to remain in saturation: VD(S)≥VCMGS-Vt

⇒VCM,max=VD-Vt

Lowest VCM is determined by minimum voltage VCS across the current source I.

i.e. VCSmin=-VSS+VCM,min+VGS (KVL)

⇒VCM,min=-VGS+VCSmin-VSS

## Differential Pair Small Signal Analysis

Ad=vo2vid ; vo2=νod2		Acm=vo2vcm

## Half Circuit Analysis for Differential-Mode νid and Common-Mode νcm

| Differential-Mode νid - Requires symmetry. - Cut circuit in half. - DM Inputs are equal and opposite. - Inputs: +νid2 (Q1), -νid2 (Q2) - Outputs: -νod2 (Q1), +νod2 (Q2) - Source node = 0V (AC virtual ground) - Analyze half with ss model. | Common-Mode νcm - Requires symmetry. - Cut circuit in half. - CM Inputs are equal and in-phase. - Inputs/outputs: νcm - Source node = 2RSource. - Analyze half with ss model. |
| --- | --- |

## Common-Mode Rejection Ratio (CMRR)

CMRR=AdAcm

# M3: Frequency Response

## Definition

Lower 3dB Frequency fL	Upper 3dB Frequency fH

Midband Voltage Gain AM=vovsig (short circuit all capacitors)