ELECENG 3EJ4

Lab Report #2

Instructor: Dr. Chen

Jasmine Dosanjh – dosanj5 – 400531879

October 5, 2025

# Part 1 Questions

# Q1. (10 Points)

(1) Step 1.2: Vo,min=-3V, Io=1.848×10-4A

Step 1.10: Vo,min=-3V, Io=2.055×10-4A

(2) Step 1.2: Ro=-2.41×106,4.07×106 Ω

Step 1.10: Ro=-6.52×106,1.58×107 Ω

# Q2. (10 Points)

(1) Vo1=4.94V, Vo2=-3.578V

(2) The above voltages correspond to the edge values of Vsig, in which the circuit functions as an amplifier:

When Vsig(min)=4.39 V⟹Vo1=Vo(max)

Vsig(max)=4.41 V⟹Vo2=Vo(min)

Therefore, these points represent the maximum and minimum output voltages of the amplifier.

# Q3. (15 Points)

(1)

The graph above has three key regions:

- Top flat region: Changing Vsig does not change Vo, meaning there is no amplification. Since Vo≈VCC, the BJT conducts no current and is in cutoff.

∆Vo≈0⟹Gain=∆Vo∆Vsig≈0

- Bottom flat region: Same as top flat region, but Vo≈-3.6V, meaning the BJT is fully on with no gain, meaning its in saturation.

- Middle steep region: A tiny change in Vsig produces a large change in Vo, meaning this is where the circuit behaves like an amplifier (BJT forward active).

Gain=∆Vo∆Vsig=large

(2) From the graph in (1), for the circuit to work as an amplifier we must find the range of the middle steep region:

- Vsig=4.4006, 4.4028 V

- Vo=4.6894, -3.175 V

(3) For Vo=0.124≈0V:

- Vsig=4.4018V

- IC2=1.85×10-4 A

(4)

# Q4. (10 Points)

(1) For low frequency of 100 Hz:

- Avo=20logVoVi=20log4.047V1mV×2=20log4.047V0.002V=66.1dB

- Phaselow=179.6°

Upper 3-dB frequency:

- Phase3dB=Phaselow-45°=134.6°

- f3dB=14402 Hz

- Avo=20logVoVi=20log2.84V0.002V=63.04dB

(2) The measured gain was Avo=63.5 dB. This is similar to the values seen above.

(3) Adjusting the frequency of W1 to equal f3dB:

For this frequency the gain is: Avo=20logVoVi=20log2.0299 V4.5672 mV=52.95B

Theoretically we expect, Avo100Hz×0.707=45dB

Thus, there is a slight error between the theoretical and measured values, but they follow the same pattern that an increased frequency in Vsig results in a lower gain, Avo.

# Part 2 Questions

# Q5. (15 Points)

(1) When VCM=0V:

- Vo=4.2499V

- VE=-0.525V

- IC2=9.10×10-5A

(2) The range of VCM to maintain the same Vo can be seen by the graph below:

This graph shows that a constant Vo occurs for VCM=-2.6, 4.6 V.

(3) The upper and lower bounds of VCM set both the transistor operation limits in the differential pair and current sink:

- Lower Bound: Limited by the need to keep the current sink Q3 on, because if VCM is too low the transistor will be in cutoff.

- Upper Bound: Limited by the need to keep the transistor pairs Q1 and Q2 in active mode because if VCM is too high the transistor will be in saturation.

(4) The measurement graphs below correspond to Steps 2.7 and 2.8.

The left graph from Step 2.7 shows that for a constant output voltage, VCM(max)=4.5, 4.7 V. The right graph from Step 2.8 shows VCM(max)=-2.7, -2.5 V. This matches the range VCM=-2.6, 4.6 V found in (2).

# Q6. (10 Points)

Based on the data in Step 2.3, the low-frequency voltage gain for VCM is:

ACM=-86.90 dB at lowest frequency f=0.1Hz.

# Part 3 Questions

# Q7. (10 Points)

(1) Based on the simulation data obtained below in Step 3.2:

Section 9.2.3 of the textbook tells us that the limitations for the differential-mode ranges is 4VT about the point of intersection of Series 1 and 2 for the DC case and VT2 for AC, where VT=0.025 V.

The intersection of the series is at V1=0.004 V.

Thus, the DC and AC input differential-mode ranges are,

- VDM=V1-4VT,  V1+4VTV=-0.096,  0.104 V

- vdm=V1-VT2,  V1+VT2V=-0.0085,  0.0162 V

(2) The upper and lower bounds of these input differential-mode ranges are determined by the range in which the two BJTs remain in active operation and share current. Around the intersection of the two collector current curves, both transistors conduct and the circuit behaves as a linear amplifier. For AC operation, if the differential-mode input exceeds ±VT/2, the circuit becomes nonlinear. For DC operation, if the range exceeds ±4VT, one transistor saturates while the other cuts off, so the amplifier action is lost.

# Q8. (10 Points)

(1) For step 3.3 the differential mode gain is Ad=19.63 dB.

(2) For low frequencies:

- Phase=-0.000665535°

Upper 3-dB frequency:

- Phase=-0.000665535°-45°=-45.00066554°

- f3dB=8332821.508 Hz

To find the Gain-Bandwidth Product, we must convert the gain from dB to the linear ratio (V/V):

Ad=20logAv⇒Av=10Ad20=1019.6320=9.5829 V/V

Thus,

GBW=Av×f3dB=9.5829VV8332821.508Hz=79852595.23Hz

(3) Comparing the following frequencies:

- CE Amplifier (Q4): f3dB=14402 Hz

- Differential Amplifier (Q8): f3dB=8332821.508 Hz

In question 8, the 3dB frequency is much greater than question 4.

(4) Based on Step 3.6, the low-frequency differential gain is Ad=22.8 dB.

# Q9. (10 Points)

CMRR=AdAcm=19.63 dB-86.90 dB=0.22589 dB