ELECENG 3EJ4

Lab Report #3

Instructor: Dr. Chen

Jasmine Dosanjh – dosanj5 – 400531879

November 2, 2025

# Part 1 Questions

# Q1. (15 Points)

(1) Below are the plots for the simulated and measured Vo vs Vsig characteristics obtained in Steps 1.2 and 1.6:

| Figure 1. Simulated Data Step 1.2 | Figure 2. Measured Data Step 1.6 |
| --- | --- |

Both graphs are very similar, as they share the same overall shape:

Let’s explain this by examining the following equation:

VBE=VB-VE=Vsig-Vo

- For the first two points of both graphs, the slope is flat because for Vsig≤-4.5V, Vo does not change. This implies that  VBE<0.7 V, and the BJT is in cutoff.

Figure 3. Simulated Data Step 1.2

Once Vo increases linearly with Vsig, VBE>0.7V and the transistor enters the active region,

(2) The DC input range for Vsig is -5V≤Vsig≤5V, but as mentioned in (1) Vo is flat for Vsig≤-4.5V. This means that the circuit does not work as a common collector in this region as the gain VoVsig=0≠1.

Thus, the DC input range for Vsig occurs where the BJT is in the active region, and that is -4.5V≤Vsig≤5V which corresponds to Vo≈-4.5, 4.5 V.

(3) According to the simulated data from Step 1.2 (refer to Figure 3), the Vsig value that results in Vo≈0 V is 0.5 V.

# Q2. (10 Points)

Based on the simulation and measurement data obtained in Steps 1.3 and 1.8, the intrinsic voltage gain Avo (magnitude and phase) at low frequency (100 Hz) is:

Step 1.3: Avo=0 dB ∠-8.47×10-5°

Figure 4. Simulated Data Step 1.3

Step 1.8: Avo=0.8 dB

Figure 5. Measured Data Step 1.8

# Part 2 Questions

# Q3. (15 Points)

(1) Based on section 8.2.4 in the textbook, the relationship to express the mirror’s output current Io as a function of IREF is given by:

IoIREF=Area of EBJ of Q2Area of EBJ of Q1

Figure 6. Simulated Data Step 2.2

For the schematic in Fig. 3, Q1 and Q2 are the same transistors (2N3906), thus the areas will be the same. This implies that Io=IREF.

(2) Based on the simulation data obtained in Step 2.2,

At IREF=0.1 mA, Io=0.000104 A ⇒Io=1.04IREF.

At IREF=1 mA, Io=0.000975 A⇒Io=0.975IREF.

(3) To generate the theoretical predictions, we know from (1) that: Io=IREF. By plugging in the values of IREF from (2),

When IREF=1 mA, we should have Io=1 mA but we have Io=0.975 mA.

When IREF=0.1 mA, we should have Io=0.1 mA but we have Io=0.104 mA.

As expected, the theoretical values are very close to the simulated ones, demonstrating almost a 1:1 relationship.

Figure 7. Simulated Data Step 2.5

Q4. (15 Points)

(1) Based on the simulation data in Step 2.5,

Input impedance Rin=389.12 Ω.

Current Gain of current mirror Ai=1.04 A/A

(2) Based on the simulation data in Step 2.6,

Output impedance Ro=1.58×106 Ω.

(3) Using the above information, we know that the relation to the h-parameters is:

Rin=h11, Ro=1/h22, Ai=h21

The linear two-port network for the current mirror is shown below:

Figure 8. Linear two Port Network

With the following parameters,

h11=389.12 Ω

h22=1.01×10-4 S

h21=1.04

h12=7.05×10-7 @ 100 Hz to 1.41×10-6 @ 200 Hz

Figure 9. Simulated Data Step 2.6

# Part 3 Questions

# Q5. (15 Points)

(1) Based on the simulation data in Step 3.2,

Voltage gain Ad=70.07 dB

(2) In step 3.6, the mismatch voltage calculated was -1.27×10-2 V which was the offset voltage applied to V2 in subsequent steps.

(3) In Step 3.8,

Voltage gain Ad=60.3 dB

This is around 10 dB lower than the simulated value in (1).

# Q6. (10 Points)

Figure 10. Simulation data Step 3.2

For low frequency fL of 100 Hz:

- MagnitudeL=6.38 V

- PhaseL=-0.518°

Figure 11. Simulation data Step 3.2

The Upper 3-dB frequency fH is the frequency at which the amplitude becomes 12=0.707 of its low frequency value, or the phase changes 45°. This gives us:

- MagnitudeH=MagnitudeL× 12=4.51 V

- fH=11207 Hz

# Q7. (10 Points)

Comparing the upper 3-dB frequencies of this differential amplifier with a current mirror load, with that from Lab 2 of a differential amplifier using resistive loads…

- Lab 3 Q6: fH=11207 Hz, Ad=70.07 dB

- Lab 2 Q8: fH=8332821 Hz, Ad=19.63 dB.

The Lab 2 frequency is much larger than the one in this Lab. This is due to the Miller Theorem, which tells us that a higher voltage gain leads to a larger Miller capacitance, Cm, which in turn results in a smaller upper 3-dB frequency. Thus since, fH∝1 Ad, the larger gain of 70.07 dB in Lab 3 results in a smaller fH, and the opposite is true of the smaller gain of 19.63 dB in Lab 2.

# Q8. (10 Points)

Thus, the gain-bandwidth products (GBW) of the differential amplifier with a current mirror load, and Lab 2’s differential amplifier using resistive loads are…

Figure 12. Simulation data Step 3.2

Lab 3 Q6: GBW=3.57×107 Hz

- Lab 2 Q8: GBW=7.95×107 Hz